Supplementary Tables

Table 1. Abbreviations

| Abbreviation / number | Full name |
| --- | --- |
| Hum | Humorus |
| Rad | Radius |
| Uln | Ulna |
| MC | Metacarpal |
| PP | Proximal Phalanx |
| MP | Middle phalanx |
| DP | Distal Phalanx |
| CMC | Carpal Metacarpal |
| MCP | Metacarpal Phalangeal |
| PIP | Proximal Inter phalangeal |
| DIP | Distal Inter phalangeal |
| IP | Inter Phalangeal |
| 1 or I | Thumb |
| 2 or II | Index finger |
| 3 or III | Middle finger |
| 4 or IV | Ring finger |
| 5 or V | Little finger |

Table 2. Anthropometric data used for calculation of the inertial properties of the segments.

| Measurement | Length (mm) (mm) |
| --- | --- |
| Forearm length | 248 |
| Stylion –MCIII length | 34 |
| MCIII-dactylion length | 162 |
| Elbow width | 65 |
| Wrist width | 55 |
| Hand width | 68 |
| Axillary arm circumference | 245 |
| Mid arm circumference | 170 |
| Wrist circumference | 194 |
| Hand circumference | 160 |

Table 3. Joint axes and their difference (Diff.) with geometrical axes

|  |  | Rotation axis direction vector (˚) |  |  |  | Pivot point on axis (mm) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Joint | Axis | X | Y | Z | Diff | X | Y | Z | Diff |
| Elbow | Flexion | -0.62 | 0.07 | -0.78 | - | 19.20 | 216.54 | 7.06 | - |
|  | Pronation | -0.07 | -1.00 | -0.06 | - | 9.54 | 26.50 | -3.31 | - |
| Wrist | Flexion | -0.98 | -0.16 | -0.07 | - | 32.26 | -9.16 | -10.61 | - |
|  | Abduction | -0.11 | 0.04 | 0.99 | - | 30.53 | -16.03 | -6.55 | - |
| CMC 1 | Flexion | -0.27 | 0.64 | -0.72 | - | 48.54 | -37.82 | -6.63 | - |
|  | Abduction | -0.78 | -0.61 | -0.13 | - | 49.16 | -39.46 | -13.15 | - |
| MCP 1 | Flexion | -0.67 | -0.07 | 0.74 | 13 | 68.92 | -64.16 | -3.54 | 3 |
| IP 1 | Flexion | -0.36 | -0.25 | 0.90 | 17 | 88.26 | -89.46 | 4.58 | 1 |
| MCP 2 | Flexion | -0.99 | -0.11 | -0.09 | 27 | 47.30 | -92.88 | 0.29 | 10 |
|  | Abduction | 0.00 | 0.42 | 0.91 | - | 47.24 | -93.18 | 0.05 | - |
| PIP 2 | Flexion | -0.99 | 0.11 | -0.07 | 6 | 52.99 | -134.30 | 2.76 | 4 |
| DIP 2 | Flexion | -0.96 | 0.17 | -0.21 | 16 | 49.47 | -155.97 | -0.29 | 5 |
| MCP 3 | Flexion | -0.99 | 0.06 | -0.08 | 25 | 31.83 | -92.51 | 0.73 | 3 |
|  | Abduction | -0.02 | -0.20 | -0.98 | - | 28.71 | -95.88 | -3.01 | - |
| PIP 3 | Flexion | -0.99 | 0.10 | -0.12 | 9 | 42.74 | -137.97 | -6.01 | 4 |
| DIP 3 | Flexion | -0.97 | 0.19 | -0.14 | 9 | 33.06 | -163.59 | -19.28 | 3 |
| MCP 4 | Flexion | -0.97 | 0.22 | -0.11 | 9 | 12.12 | -86.07 | -1.98 | 3 |
|  | Abduction | -0.26 | -0.22 | -0.94 | - | 7.05 | -85.94 | -2.27 | - |
| PIP 4 | Flexion | -0.88 | 0.36 | -0.32 | 23 | -2.65 | -125.60 | 1.48 | 18 |
| DIP 4 | Flexion | -0.92 | 0.18 | -0.35 | 23 | -2.50 | -152.19 | -6.44 | 8 |
| MCP 5 | MCP5f | -0.91 | 0.33 | -0.24 | 24 | -2.44 | -79.11 | -3.20 | 6 |
|  | MCP5a | -0.01 | -0.31 | -0.95 | - | -6.01 | -79.66 | 1.68 | - |
| PIP 5 | PIP5 | -0.84 | 0.28 | -0.47 | 28 | -13.57 | -112.38 | 0.66 | 5 |
| DIP 5 | DIP5 | -0.91 | 0.28 | -0.30 | 19 | -17.20 | -129.98 | -9.24 | 14 |

Table 4. Coordinates of landmarks (relative to ulnar styloid in mm) .* indicates that the data are not available.

| Landmark | Name | X | Y | Z |
| --- | --- | --- | --- | --- |
| EL | Lateral epicondile humerus | 13.1 | 236.6 | 24.5 |
| EM | Medial epicondile humerus | -13.1 | 221.0 | -24.5 |
| US | Ulnar styloid | 0.0 | 0.0 | 0.0 |
| RS | Radial styloid | 51.7 | 1.7 | 0.0 |
| MC1b | Base MC I | 61.8 | -22.6 | -14.1 |
| MC1h | Head MC I | 80.3 | -61.6 | -3.2 |
| MCP1u | Ulnar side MCP I joint | 70.9 | -67.0 | 3.5 |
| MCP1r | Radial side MCP I joint | 77.7 | -62.7 | -18.1 |
| PP1h | Head PP I | 93.8 | -84.5 | 5.2 |
| IP1u | Ulnar side IP I joint | 84.7 | -90.5 | 8.4 |
| IP1r | Radial side IP I joint | 94.4 | -85.7 | -8.3 |
| DP1h | Head DP I | 104.4 | -114.0 | -1.4 |
| MC2b | Base MC II | 49.1 | -36.0 | 5.5 |
| MC2hh | Head MC II | 56.2 | -87.9 | 7.8 |
| PP2h | Head PP II | 46.8 | -131.6 | 8.5 |
| PIP2u | Ulnar side PIP II joint | 36.6 | -131.2 | 2.3 |
| PIP2r | Radial side PIP II joint | 55.5 | -135.5 | 1.5 |
| MP2h | MP II head | 43.8 | -155.7 | 3.5 |
| DIP2u | Ulnar side DIP II joint | 35.9 | -154.5 | -3.8 |
| DIP2r | Radial side DIP II joint | 53.9 | -158.2 | -0.8 |
| DP2h | DP II head | * | * | * |
| MC3b | Base MC III | 35.5 | -28.7 | 9.1 |
| MC3h | Head MC III | 34.1 | -82.7 | 10.3 |
| PP3h | Head PP III | 23.5 | -135.8 | -1.0 |
| PIP3u | Ulnar side PIP III joint | 15.0 | -133.5 | -8.2 |
| PIP3r | Radial side PIP III joint | 32.9 | -136.9 | -5.2 |
| MP3h | MP III head | 21.0 | -160.8 | -14.6 |
| DIP3u | Ulnar side DIP III joint | 14.8 | -155.1 | -19.3 |
| DIP3r | Radial side DIP III joint | 30.5 | -161.4 | -18.8 |
| DP3h | DP III head | 20.8 | -179.6 | -32.6 |
| MC4b | Base MC IV | 11.5 | -25.2 | 0.6 |
| MC4h | Head MC IV | 7.6 | -79.4 | 7.6 |
| PP4h | Head PP IV | -6.4 | -121.8 | 5.4 |
| PIP4u | Ulnar side PIP IV joint | -12.5 | -122.3 | 0.8 |
| PIP4r | Radial side PIP IV joint | 3.4 | -127.5 | 0.8 |
| MP4h | MP IV head | -6.7 | -152.4 | -2.8 |
| DIP4u | Ulnar side DIP IV joint | -12.5 | -150.6 | -8.5 |
| DIP4r | Radial side DIP IV joint | 2.0 | -153.2 | -4.6 |
| DP4h | DP IV head | -4.9 | -176.6 | -13.7 |
| MC5b | Base MC V | 3.4 | -36.9 | -4.0 |
| MC5h | Head MC V | -7.9 | -71.6 | 3.4 |
| PP5h | Head PP V | -16.4 | -110.7 | 2.9 |
| PIP5u | Ulnar side PIP V joint | -24.8 | -114.1 | -5.3 |
| PIP5r | Radial side PIP V joint | -11.4 | -115.9 | -0.3 |
| MP5h | MP V head | -21.8 | -133.3 | -5.9 |
| DIP5u | Ulnar side DIP V joint | -25.4 | -129.9 | -13.0 |
| DIP5r | Radial side DIP V joint | -12.5 | -129.9 | -9.8 |
| DP5h | DP V head | -16.4 | -150.2 | -12.6 |

Table 5. Repeated landmarks positions (in mm) and difference (Diff).

|  | First measurement |  |  | Second measurement |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | X | Y | Z | X | Y | Z | Diff |
| MC3b | 35.53 | -28.72 | 9.08 | 36.81 | -33.96 | 10.43 | 5.56 |
| PP3h | 23.53 | -135.77 | -1.02 | 24.18 | -134.46 | -1.38 | 1.50 |
| MP3h | 20.97 | -160.82 | -14.56 | 20.88 | -160.18 | -14.36 | 0.67 |
| DP3h | 20.80 | -179.63 | -32.59 | 20.37 | -178.89 | -32.53 | 0.85 |
| MC4b | 11.50 | -25.22 | 0.63 | 9.54 | -28.68 | 0.14 | 4.00 |
| MC4h | 7.62 | -79.38 | 7.61 | 11.31 | -82.39 | 9.79 | 5.23 |
| PP4h | -6.39 | -121.83 | 5.43 | -4.81 | -122.71 | 5.58 | 1.82 |
| MP4h | -6.66 | -152.45 | -2.78 | -6.48 | -151.02 | -2.34 | 1.50 |
| DP4h | -4.90 | -176.55 | -13.67 | -5.24 | -176.26 | -12.74 | 1.03 |
| MC5b | 3.44 | -36.94 | -3.96 | 4.32 | -42.92 | -3.23 | 6.09 |
| MC5h | -7.91 | -71.65 | 3.40 | -7.56 | -73.77 | 4.01 | 2.24 |
| PP5h | -16.37 | -110.68 | 2.94 | -19.30 | -110.47 | 2.50 | 2.97 |
| MP5h | -21.83 | -133.31 | -5.92 | -21.16 | -132.95 | -5.33 | 0.96 |
| DP5h | -16.43 | -150.16 | -12.64 | -15.65 | -149.57 | -14.52 | 2.11 |
| MEAN |  |  |  |  |  |  | 2.61 |
| STD |  |  |  |  |  |  | 1.87 |

Table 6. Cylinder curves

| Joint | Direction x | y | z | Point x | y | Z | R (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCPI | -0.662 | -0.290 | 0.691 | 71.95 | -62.00 | -5.44 | 6.66 |
| IPI | -0.618 | -0.177 | 0.766 | 90.00 | -86.88 | -0.79 | 6.55 |
| MCPII | -0.877 | -0.412 | 0.245 | 45.50 | -89.28 | -2.03 | 12.53 |
| PIPII | -0.999 | 0.045 | 0.000 | 45.08 | -135.71 | 2.21 | 2.96 |
| DIPII | -1.002 | 0.009 | 0.000 | 43.12 | -158.16 | 0.82 | 1.45 |
| MCPIII | -0.997 | -0.072 | 0.001 | 28.15 | -90.10 | 2.60 | 8.27 |
| PIPIII | -0.990 | 0.131 | 0.044 | 23.15 | -137.11 | -7.44 | 4.47 |
| DIPIII | -0.986 | 0.168 | 0.000 | 26.86 | -164.36 | -16.07 | 2.19 |
| MCPIV | -0.994 | 0.108 | 0.000 | 6.86 | -84.68 | 0.41 | 5.63 |
| PIPIV | -0.992 | 0.130 | 0.000 | -2.52 | -128.10 | -0.05 | 3.22 |
| DIPIV | -1.007 | 0.000 | 0.000 | -5.94 | -154.92 | -6.61 | 1.54 |
| MCPV | -0.780 | 0.627 | 0.000 | -3.33 | -77.49 | -2.49 | 7.73 |
| PIPV | -0.986 | 0.182 | 0.000 | -18.17 | -114.00 | -1.96 | 2.53 |
| DIPV | -0.986 | 0.168 | 0.000 | -20.66 | -130.94 | -8.14 | 1.00 |

Table 7. Position of muscle points (Po.  I = insertion, O= origin, via= via point. Additions: d = distal, p= proximal, u= ulnar, r= radial) and related segments (Seg). Abb=Abbreviation. x= Missing data. Extensor wrap points (Po.) are illustrated in supplementary figures.

| Muscle | Abb. | Po. | Seg. | Position (mm) |  |  | Info |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | X | Y | Z |  |
| WRIST |  |  |  |  |  |  |  |
| Extensor carpi radialis longus | ECRL | I | MC2 | 50.5 | -36.8 | 2.8 |  |
|  |  | Via | Radius | 43.9 | 7.4 | 6.4 |  |
|  |  | Od | Humerus | 31.3 | 213.5 | 12.5 |  |
|  |  | Op | Humerus | 19.4 | 242.7 | 13.6 |  |
| Extensor carpi radialis brevis | ECRB | I | MC3 | 35.1 | -39.2 | 6.8 |  |
|  |  | Via | Radius | 39.9 | 8.1 | 8.0 |  |
|  |  | Od | Radius | 31.1 | 134.2 | 14.1 |  |
|  |  | Op | Humerus | 30.6 | 207.2 | 17.2 |  |
| Extensor carpi ulnaris | ECU | I | MC5 | 0.1 | -38.4 | -7.6 |  |
|  |  | Via | Radius | 7.8 | 1.4 | -0.4 |  |
|  |  | Od | Ulna | 4.8 | 92.9 | 12.4 |  |
|  |  | Op | Humerus | 17.6 | 203.4 | 21.7 |  |
| Palmaris longus | PL | I | MC3 | 36.3 | -82.3 | -10.3 |  |
|  |  | I | MC3 | 6.2 | -73.7 | -16.2 |  |
|  |  | I | MC3 | 49.9 | -25.6 | -23.7 |  |
|  |  | I | MC3 | 51.2 | -31.7 | -22.9 |  |
|  |  | Via | MC3 | 31.7 | -22.5 | -24.5 |  |
|  |  | Via | Radius | 27.5 | 0.5 | -26.6 |  |
|  |  | Od | Ulna | 33.2 | 225.9 | -1.7 |  |
|  |  | Op | Humerus | -5.6 | 201.7 | -20.1 |  |
| Flexor carpi ulnaris | FCU | I | MC3 | 17.5 | -10.1 | -23.6 |  |
|  |  | I | MC5 | 9.1 | -24.6 | -24.2 |  |
|  |  | Via | Radius | 8.0 | 12.7 | -20.0 |  |
|  |  | Od | Ulna | 2.7 | 92.6 | 6.8 |  |
|  |  | Op | Ulna | 0.4 | 212.3 | 9.9 |  |
|  |  | O | Humerus | -14.7 | 220.0 | -15.4 |  |
| Flexor carpi radialis | FCR | I | MC2 | 45.9 | -47.5 | -5.8 |  |
|  |  | Via | Radius | 44.1 | -12.2 | -28.4 |  |
|  |  | O | Humerus | -5.3 | 224.3 | -25.2 |  |
| Supinator | Sup | Id | Radius | 28.7 | 123.4 | 17.7 |  |
|  |  | Ip | Radius | 17.1 | 175.2 | -4.7 |  |
|  |  | O | Humerus | 25.4 | 220.1 | 18.8 |  |
| Pronator Quadratus | PrQ | Id | Radius | 45.7 | 13.9 | -9.4 |  |
|  |  | Ip | Radius | 40.5 | 49.9 | 0.0 |  |
|  |  | Od | Ulna | 6.1 | 15.5 | -10.1 |  |
|  |  | Op | Ulna | 4.9 | 53.9 | -7.4 |  |
| Pronator Teres | PrT | Id | Radius | 39.2 | 88.8 | 10.8 |  |
|  |  | Ip | Radius | 31.5 | 128.3 | 13.3 |  |
|  |  | O | Humerus | -7.5 | 224.5 | -25.0 |  |
| THUMB |  |  |  |  |  |  |  |
| Extensor wrap 1 | Extwrap1 | T1 | DP1 | 106.8 | -94.6 | -7.5 |  |
|  |  | T2 | PP1 | 90.3 | -76.7 | -1.1 |  |
|  |  | T3 | PP1 | 81.7 | -66.0 | -16.4 |  |
|  |  | T4 | PP1 | 71.0 | -68.5 | 1.7 |  |
|  |  | T5 | MC1 | 77.4 | -53.0 | -6.7 |  |
|  |  | T6 | MC1 | 73.2 | -53.4 | -0.7 |  |
|  |  | T7 | MC1 | 65.9 | -29.3 | -13.5 |  |
|  |  | T8 | MC1 | 59.0 | -31.3 | -4.4 |  |
|  |  | T9 | Radius | 52.2 | 5.5 | -4.7 |  |
|  |  | T10 | Radius | 36.6 | -3.4 | 5.3 |  |
| Extensor pollicis longus | EPL | Via | Ulna | 16.9 | 46.6 | 8.7 |  |
|  |  | O | Ulna | 1.7 | 127.1 | 18.2 |  |
| Extensor pollicis brevis | EPB | Via | Radius | 52.2 | 4.6 | -6.3 |  |
|  |  | Via | Ulna | 34.6 | 33.7 | 15.2 |  |
|  |  | O | Ulna | 19.9 | 75.0 | 6.0 |  |
| Abductor pollicis longus | AbPL | I | MC1 | 65.9 | -29.7 | -17.6 |  |
|  |  | Via | Radius | 51.3 | 5.8 | -5.7 |  |
|  |  | Via | Ulna | 44.6 | 37.3 | 6.8 |  |
|  |  | O | Ulna | 11.0 | 143.6 | 15.9 |  |
| Abductor pollicis brevis | AbPB | I | PP1 | 81.4 | -65.0 | -15.4 |  |
|  |  | Od | MC3 | 32.2 | -44.5 | -16.4 |  |
|  |  | Om | MC3 | 28.3 | -30.8 | -21.8 |  |
|  |  | Op | MC3 | 34.1 | -20.6 | -23.4 |  |
| Flexor pollicis brevis -superficiales | FPBs FPBs | I | PP1 | 74.0 | -68.1 | -18.9 |  |
|  |  | O | MC3 | 29.1 | -42.3 | -17.1 |  |
| Opponens pollicis | OPP OPP | Id | MC1 | 76.6 | -60.9 | -16.2 |  |
|  |  | Ip | MC1 | 63.3 | -29.3 | -18.9 |  |
|  |  | Od | MC3 | 38.9 | -22.3 | -23.6 |  |
|  |  | Op | MC3 | 29.7 | -29.3 | -20.7 |  |
| Flexor pollicis longus | FPL | I | DP1 | 87.5 | -96.9 | -1.0 |  |
|  |  | Via | PP1 | 82.0 | -88.2 | -4.7 |  |
|  |  | Via | PP1 | 74.1 | -78.5 | -9.1 |  |
|  |  | Via | MC1 | 59.4 | -63.6 | -15.6 |  |
|  |  | Via | MC3 | 37.9 | -22.3 | -20.9 |  |
|  |  | Od | Radius | 33.1 | 49.9 | -2.3 |  |
|  |  | Op | Radius | 19.4 | 154.1 | 0.5 |  |
| Adductor pollicis – transverse | AdPt AdPt | I | PP1 | 68.1 | -74.2 | -5.7 |  |
|  |  | Od | MC3 | 26.4 | -67.7 | -2.3 |  |
|  |  | Op | MC3 | 28.6 | -37.2 | -9.4 |  |
| Adductor pollicis – oblique | AdPo | I | PP1 | 68.1 | -74.2 | -5.7 |  |
|  |  | Od | MC3 | 28.6 | -37.2 | -9.4 |  |
|  |  | Op | MC3 | 31.9 | -32.1 | -13.2 |  |
| Flexor pollicis brevis - deep | FPBd | I | PP1 | 70.5 | -71.5 | -15.8 |  |
|  |  | O | MC3 | 31.9 | -32.1 | -13.2 |  |
|  |  | O | MC3 | 9.1 | 124.3 | 118.6 |  |
| Index finger |  |  |  |  |  |  |  |
| Extensor wrap 2 | ExtWrap2 | I1 | DP2 | 42.2 | -164.4 | 1.8 |  |
|  |  | I2 | MP2 | 45.0 | -151.9 | 3.6 |  |
|  |  | I3 | MP2 | 44.4 | -136.8 | 6.9 |  |
|  |  | I4 | PP2 | 45.4 | -130.8 | 7.1 |  |
|  |  | I5 | PP2 | 52.5 | -133.3 | 4.6 |  |
|  |  | I6 | PP2 | 38.9 | -135.3 | 4.2 |  |
|  |  | I7 | PP2 | 47.1 | -106.6 | 9.1 |  |
|  |  | I8 | PP2 | 51.6 | -99.8 | 8.7 |  |
|  |  | I9 | PP2 | 59.4 | -100.0 | 2.1 |  |
|  |  | I10 | PP2 | 39.1 | -102.0 | 3.0 |  |
|  |  | I11 | MC2 | 50.7 | -84.0 | 11.3 |  |
|  |  | I12 | MC2 | 44.8 | -85.1 | 10.9 |  |
|  |  | I12 | MC2 | 44.8 | -85.9 | 10.8 |  |
|  |  | I13 | MC2 | 43.9 | -34.0 | 7.8 |  |
|  |  | I14 | MC2 | 37.2 | -35.3 | 8.7 |  |
|  |  | I15 | Radius | 30.5 | 1.9 | 9.4 |  |
|  |  | I16 | Radius | 26.6 | 3.9 | 8.0 |  |
| Extensor digitorum | EDC | Via | Radius | 16.0 | 86.6 | 14.3 |  |
|  |  | O | Humerus | 26.3 | 223.9 | 21.3 | Common origin |
| Extensor digitorum II | EDC2 | Via | Radius | 25.1 | 89.2 | 15.3 |  |
|  |  | O | Ulna | 33.9 | 212.4 | 24.5 |  |
| Extensor indicis | EI | O | Ulna | 15.6 | 43.8 | 2.7 |  |
|  |  | O | Ulna | 1.6 | 99.6 | 14.1 |  |
| Flexor digitorum superficiales II | FDS2 | Iu | MP2 | 42.7 | -144.2 | -4.9 |  |
|  |  | Ir | MP2 | 48.0 | -148.1 | -3.4 |  |
|  |  | Via | PP2 | 46.5 | -129.7 | -4.3 | PIP |
|  |  | Via | PP2 | 47.2 | -114.3 | -3.7 | Split |
|  |  | Via | PP2 | 46.5 | -104.4 | -8.2 | Mcp |
|  |  | Via | MC2 | 43.5 | -84.8 | -9.1 |  |
|  |  | Via | MC3 | 31.3 | -30.7 | -20.2 |  |
| Flexor digitorum superficiales common | FDS | Od | Radius | 39.1 | 85.5 | -7.1 | Common origin |
|  |  | Om | Radius | x | x | x |  |
|  |  | Op | Radius | x | x | x |  |
|  |  | O | Ulna | 2.2 | 196.9 | -8.5 |  |
|  |  | O | Humerus | -5.3 | 224.3 | -25.2 |  |
| Flexor digitorum profundus II | FDP2 | I | DP2 | 45.5 | -163.2 | -5.7 |  |
|  |  | Via | MP2 | 45.7 | -149.4 | -4.8 |  |
|  |  | Via | MP2 | 45.6 | -138.7 | -6.3 |  |
|  |  | Via | PP2 | 47.1 | -127.2 | -1.4 |  |
|  |  | Via | PP2 | 47.9 | -106.1 | -3.6 |  |
|  |  | Via | MC2 | 44.1 | -86.2 | -6.7 |  |
|  |  | Via | MC2 | 36.9 | -55.2 | -10.6 | Origin of Carpals lumbricales |
|  |  | Via | MC2 | 31.6 | -27.9 | -16.9 |  |
| Lumbricales | LU1 | I | PP2 | 59.3 | -103.0 | -2.5 |  |
|  |  | O | MC2 | 36.9 | -55.2 | -10.6 |  |
| Flexor digitorum profundus common | FDP | Od | Radius | 25.3 | 72.3 | 2.4 | Common origin |
|  |  | Op | Radius | 15.5 | 153.7 | 3.8 |  |
|  |  | Od | Ulna | 2.9 | 74.0 | 0.7 |  |
|  |  | Om | Ulna | 10.8 | 134.1 | 11.5 |  |
|  |  | Op | Ulna | 10.2 | 206.8 | 19.8 |  |
| Palmar Interosseus 2 | PI2 | I | PP2 | 39.7 | -103.9 | -1.8 |  |
|  |  | Via | MC2 | 38.4 | -88.3 | -4.5 |  |
|  |  | Od | MC2 | x | x | x |  |
|  |  | Op | MC2 | 39.1 | -45.5 | -4.7 |  |
|  |  | O | MC3 | 33.7 | -45.1 | -5.0 |  |
| Dorsal Interosseus | DI1 | I | PP2 | 55.8 | -107.9 | 0.5 |  |
|  |  | Via | MC2 | 57.9 | -88.3 | -4.7 |  |
|  |  | Od | MC2 | 46.9 | -71.0 | -2.0 |  |
|  |  | Op | MC2 | 46.4 | -49.7 | -5.3 |  |
|  |  | Od | MC1 | 61.7 | -56.7 | -6.0 |  |
|  |  | Op | MC1 | 53.0 | -39.2 | -6.2 |  |
| Middle finger | III |  |  |  |  |  |  |
| Extensor Wrap III | ExtWrap3 | M1 | DP3 | 22.1 | -169.3 | -19.2 |  |
|  |  | M2 | MP3 | 24.3 | -156.9 | -8.4 |  |
|  |  | M3 | MP3 | x | x | x |  |
|  |  | M4 | PP3 | 22.7 | -135.2 | -1.6 |  |
|  |  | M5 | PP3 | 29.8 | -138.1 | -3.8 |  |
|  |  | M6 | PP3 | 15.0 | -137.7 | -5.9 |  |
|  |  | M7 | PP3 | 26.8 | -109.1 | 6.3 |  |
|  |  | M8 | PP3 | 26.9 | -102.7 | 7.5 |  |
|  |  | M9 | PP3 | 36.5 | -103.9 | 0.9 |  |
|  |  | M10 | PP3 | 18.1 | -102.3 | -2.4 |  |
|  |  | M11 | MC3 | 28.6 | -70.9 | 9.7 |  |
|  |  | M12 | MC3 | 27.7 | -29.8 | 9.0 |  |
|  |  | M13 | Radius | 23.2 | 1.0 | 7.8 |  |
| Flexor digitorum Superficiales 3 | FDS3 | Iu | MP3 | 24.0 | -153.2 | -15.7 |  |
|  |  | Ir | MP3 | 28.6 | -153.9 | -14.3 |  |
|  |  | Via | PP3 | 23.9 | -130.2 | -11.0 |  |
|  |  | Via | PP3 | 27.2 | -111.2 | -9.2 |  |
|  |  | Via | PP3 | 28.1 | -105.2 | -9.7 |  |
|  |  | Via | MC3 | 28.8 | -83.1 | -9.2 |  |
|  |  | Via | MC3 | 28.2 | -32.1 | -20.2 |  |
| Flexor digitorum profundus III | FDP3 | I | DP3 | 24.6 | -169.8 | -29.1 |  |
|  |  | Via | MP3 | 27.8 | -153.9 | -15.8 | Distal part |
|  |  | Via | MP3 | 25.3 | -139.6 | -14.3 | Proximal part |
|  |  | Via | PP3 | 25.9 | -129.3 | -9.0 | Distal part |
|  |  | Via | PP3 | 27.1 | -104.2 | -5.1 | Proximal part |
|  |  | Via | MC3 | 29.0 | -85.0 | -6.6 | Head |
|  |  | Via | MC3 | 27.4 | -57.1 | -8.3 | O Lumbr |
|  |  | Via | MC3 | 26.7 | -25.7 | -13.9 | Carpals |
| Lumbricales 2 | LU2 | I | PP3 | 38.3 | -101.6 | -3.1 |  |
|  |  | Via | MC3 | 27.4 | -57.1 | -8.3 |  |
| Dorsal Interosseus 3 | DI3 | I | PP3 | 18.8 | -103.6 | -4.6 |  |
|  |  | Via | MC3 | 19.7 | -84.6 | -1.2 |  |
|  |  | Od | MC3 | 25.3 | -73.5 | -2.2 |  |
|  |  | Op | MC3 | 26.6 | -47.4 | -3.6 |  |
|  |  | Od | MC4 | 17.5 | -72.1 | -0.4 |  |
|  |  | Op | MC4 | 20.4 | -49.9 | -0.8 |  |
| Dorsal interosseus 2 | DI2 | I | PP3 | 35.6 | -104.6 | -0.5 |  |
|  |  | Via | MC3 | 37.3 | -90.2 | 0.0 |  |
|  |  | Od | MC2 | 40.4 | -78.3 | 3.9 |  |
|  |  | Op | MC2 | 38.5 | -46.0 | 5.0 |  |
|  |  | Od | MC3 | 31.0 | -74.9 | -2.0 |  |
|  |  | Op | MC3 | 32.8 | -48.2 | -3.0 |  |
| Ring finger | IV |  |  |  |  |  |  |
| Extensor Wrap 4 | ExtWrap4 | R1 | DP4 | -8.4 | -160.0 | -7.0 |  |
|  |  | R2 | MP4 | -6.3 | -148.2 | -2.2 |  |
|  |  | R3 | MP4 | -5.3 | -131.2 | 2.5 |  |
|  |  | R4 | PP4 | -2.9 | -122.9 | 3.8 |  |
|  |  | R5 | PP4 | 4.8 | -129.2 | 1.3 |  |
|  |  | R6 | PP4 | -7.6 | -126.5 | -0.1 |  |
|  |  | R7 | PP4 | 3.6 | -99.2 | 6.2 |  |
|  |  | R8 | PP4 | 3.8 | -94.6 | 6.2 |  |
|  |  | R9 | PP4 | 12.4 | -98.9 | 0.7 |  |
|  |  | R10 | PP4 | -1.8 | -92.5 | -0.7 |  |
|  |  | R11 | MC4 | 12.9 | -74.7 | 8.0 |  |
|  |  | R12 | MC4 | 1.3 | -78.0 | -6.1 |  |
|  |  | R13 | MC4 | 3.4 | -67.1 | 3.5 |  |
|  |  | R14 | MC4 | 17.1 | -30.7 | 5.1 |  |
|  |  | R15 | MC4 | 26.4 | 6.6 | 5.1 |  |
| Flexor Digitorum Superficiales IV | FDS4 | Iu | MP4 | -4.1 | -142.1 | -8.4 |  |
|  |  | Ir | MP4 | -1.1 | -140.1 | -7.0 |  |
|  |  | Via | PP4 | 1.9 | -119.6 | -4.3 | PIP |
|  |  | Via | PP4 | 6.2 | -105.0 | -6.2 | Split |
|  |  | Via | PP4 | 8.1 | -98.5 | -8.4 | Mcp |
|  |  | Via | MC4 | 13.0 | -80.5 | -13.0 |  |
|  |  | Via | MC3 | 25.3 | -30.2 | -20.1 | Carpals |
| Flexor Digitorum Profundus IV | FDP4 | I | DP4 | -3.0 | -163.4 | -13.0 |  |
|  |  | Via | MP4 | -1.5 | -146.7 | -9.1 | Distal part |
|  |  | Via | MP4 | -0.7 | -134.1 | -8.7 | Proximal part |
|  |  | Via | PP4 | 3.0 | -121.2 | -3.7 | Distal part |
|  |  | Via | PP4 | 6.1 | -98.3 | -5.3 | Proximal part |
|  |  | Via | MC4 | 13.7 | -79.2 | -9.2 |  |
|  |  | Via | MC4 | 18.0 | -58.6 | -11.5 |  |
|  |  | Via | MC3 | 24.4 | -25.2 | -14.5 | Carpals |
| Palmar Interosseus 3 | PI3 | I | PP4 | 12.1 | -100.3 | -0.3 |  |
|  |  | Via | MC4 | 16.5 | -84.4 | -10.0 |  |
|  |  | Od | MC4 | 13.0 | -59.4 | -4.2 |  |
|  |  | Op | MC4 | 16.1 | -44.0 | -8.1 |  |
| Dorsal Interosseus 4 | DI4 | I | PP4 | -1.8 | -93.9 | -1.0 |  |
|  |  | Via | MC4 | 3.6 | -77.8 | -4.2 |  |
|  |  | Od | MC5 | 3.8 | -64.4 | -1.1 |  |
|  |  | Op | MC5 | 10.7 | -47.3 | -5.0 |  |
|  |  | Od | MC4 | 10.5 | -68.8 | -4.3 |  |
|  |  | Op | MC4 | 11.5 | -45.6 | -6.3 |  |
| Little finger | V |  |  |  |  |  |  |
| Extensor Wrap V | ExtWrap5 | L1 | DP5 | -20.7 | -134.6 | -7.1 |  |
|  |  | L2 | MP5 | -21.5 | -128.0 | -5.3 |  |
|  |  | L3 | MP5 | -21.2 | -115.5 | 1.0 |  |
|  |  | L4 | PP5 | -19.2 | -109.7 | 2.3 |  |
|  |  | L5 | PP5 | -14.8 | -111.4 | 3.3 |  |
|  |  | L6 | PP5 | -24.0 | -110.1 | -2.7 |  |
|  |  | L7 | PP5 | -12.9 | -87.8 | 4.3 |  |
|  |  | L8 | PP5 | -3.2 | -90.5 | -1.4 |  |
|  |  | L9 | PP5 | -18.4 | -84.5 | -5.0 |  |
|  |  | L10 | PP5 | -9.9 | -82.9 | 4.3 |  |
|  |  | L11 | MC5 | -0.6 | -65.6 | 3.3 |  |
|  |  | L12 | MC5 | -7.1 | -64.5 | -0.8 |  |
|  |  | L13 | MC5 | 10.1 | -26.6 | -3.1 |  |
|  |  | L14 | MC5 | 8.9 | -25.6 | -4.8 |  |
|  |  | L15 | Radius | 15.3 | 2.0 | 1.0 |  |
|  |  | L16 | Radius | 12.8 | 1.9 | -2.9 |  |
| Extensor Digiti Minimi | EDM | Via | Ulna | 4.4 | 79.6 | 11.8 |  |
|  |  | O | Humerus | 8.2 | 166.7 | 15.7 |  |
| Extensor Digiti | EDM4 | O ring | Ulna | 3.0 | 105.6 | 14.8 | Origin to ring finger |
| Flexor Digiti Minimi | FDM | I | PP5 | 14.7 | -21.0 | -16.0 |  |
|  |  | O | MC3 | -13.4 | -92.3 | -11.3 |  |
| Abductor Digiti Minimi | ADM | I | PP5 | -15.2 | -86.3 | -9.1 |  |
|  |  | O | MC3 | 12.0 | -14.0 | -26.1 |  |
| Opponens Digiti Minimi | ODM | Id | MC5 | -12.1 | -71.2 | -9.1 |  |
|  |  | Ip | MC5 | 0.3 | -41.3 | -13.3 |  |
|  |  | O | MC3 | 27.1 | -32.9 | -21.6 |  |
| Flexor Digitorum Superficiales V | FDS5 | Iu | MP5 | -19.4 | -121.5 | -10.4 |  |
|  |  | Ir | MP5 | -16.4 | -121.2 | -8.1 |  |
|  |  | Via | PP5 | -13.3 | -106.3 | -4.1 | PIP |
|  |  | Via | PP5 | -10.8 | -99.1 | -5.3 | Split |
|  |  | Via | PP5 | -4.9 | -88.4 | -10.1 | MCP |
|  |  | Via | MC5 | 6.9 | -76.5 | -10.4 |  |
|  |  | Via | MC3 | 22.6 | -28.9 | -21.4 | Carpals |
| Flexor Digitorum | FDP5 | I | DP5 | -15.1 | -137.9 | -11.5 |  |
|  |  | Via | MP5 | -16.1 | -124.7 | -10.6 |  |
|  |  | Via | MP5 | -15.6 | -116.5 | -8.7 |  |
|  |  | Via | PP5 | -20.8 | -102.7 | 0.1 | Distal part |
|  |  | Via | PP5 | -8.0 | -92.3 | -5.3 | Proximal part |
|  |  | Via | MC5 | 2.1 | -80.9 | -8.6 |  |
|  |  | Via | MC5 | 15.1 | -60.2 | -10.0 | O lumbr 4 |
|  |  | Via | MC3 | 35.8 | -26.6 | -13.9 | Carpals |
| Palmar Interosseus 4 | PI4 | I | PP5 | -4.0 | -91.8 | -1.8 |  |
|  |  | Via | MC5 | 3.3 | -78.8 | -7.4 |  |
|  |  | Od | MC5 | 3.2 | -53.4 | -9.6 |  |
|  |  | Opu | MC5 | 20.8 | -26.8 | -16.3 |  |
|  |  | Opr | MC5 | 15.0 | -38.8 | -14.4 |  |

Figure 1. Thumb extensor web

Figure 2. Index finger extensor web

Figure 3. Middle finger extensor web

Figure 4. Ring finger extensor web

Figure 5. Little finger extensor web

Figure 6. Cylindrical curves (red) and digitized points (blue and green) and kinematics joint axes (dashed lines) in two different perspectives (a, b). Dimensions are in mm.

Figure 7. Dissected joints